Blind Source Separation of Inverse Square Law Signals with no Prior

# Alexander M. Glandon*, Khan M. Iftekharuddin
Electrical and Computer Engineering, Old Dominion University

# 5115 Hampton Boulevard, Norfolk, VA, USA 23529

Abstract

This work develops a blind source separation algorithm for unmixing signals received from sources at different locations arriving at a sensor array. The approach is based on the geometry of the propagation of signals that decay with inverse square of the distance from each transmitter. A difference in our approach is that we don't make any prior assumptions about the signal statistics. This will allow the method to be used for a very general class of signals.  This approach works for waves where the speed of propagation is large relative to the sensor array dimension. This approach will apply to multisource audio separation, and will also for electromagnetic signal separation with possibly larger arrays. We also see potential applications in medical sensing, for example ultrasound. For more distant EM signals, we perform an analysis of the operating limits of the method for different receiver arrays configurations. For example, given a SNR of 80dB, we can use an array to separate sources with no prior knowledge and obtain a root mean squared error of 0.000291 for 5 signals with power (mean squared amplitude) of 1 unit.

# Keywords: blind source separation, cocktail party problem, inverse square law signal

# INTRODUCTION

Blind source separation is a problem where multiple signals transmitted by multiple sources arrive at a receiver. The goal is to take the signal that is received from the combination of the sources and to determine what the original unmixed source signals are with the minimal error. Two approaches to this problem have been attempted in past research. Some groups have used a non-negative matrix factorization approach. This means looking at the receiver signal at multiple receivers such as in an array of sensors, and decomposing the measurement matrix into a transmission matrix and signal loss matrix. Our method improves upon this approach by using a more realistic model of amplitude mixing instead of power mixing. Existing methods use prior constraints on the transmitted signals to search for the unique solution from a space of low rank matrix factorization. We do not assume any preconditions on the transmitted signal, either signal statistics or cross correlation statistics between signals. We consider that the signals decay with inverse square loss, and use this knowledge to convert the underdetermined low rank matrix factorization into an overdetermined factorization problem given the receiver positions. There is other work using the receiver positions, which is different from our approach. These other methods use angle of arrival estimation to separate sources [1]. One advantage of our approach is that it will work if multiple sources are transmitting from the same direction given that the positions are different. Another advantage of our approach is that it can be applied to signals with high velocity, such as when the photon is the carrier waves. This is because angle of arrival are typically calculated from autocorrelation and time of arrival differences between different sensors in the receiver array, which would require slow carrier waves like acoustic waves. Our model works for both radio and sound. Our model is also simple to understand and gives good convergence as we demonstrate in the results.

# Background

First, we will discuss existing matrix factorization approaches to blind source separation. We will review what prior constraints these works place on the transmitted signals. The matrix factorization approach is based on the factorization in eq. 1. The transmitted signals are given in Q, where Qn,t is the transmitted signal amplitude from source n at time t. Note that without loss of generality this approach can also index the frequency f similarly to the time t. The path loss due to inverse square signal decay is given as U, where Um,n is the loss from transmitter location n to receiver location m. The received combined signals are given in P, where Pm,t is the received combined signal at receiver m, at time t.

P=UQ                                                                                   (1)

The dimensions of P are M×T, the dimensions of U are M×N, and the dimensions of Q are N×T. This factorization, given the receiver power P, is not unique. It can be solved by finding a low rank matrix factorization of P into U and Q, since typically the number of received samples in time T and the number of receivers M are both larger than the inner dimension of the factorization given by the number of transmitters N. Because the subspace of rank N has infinitely many representations or possible bases that can span the space, the factorization is not unique.

Other authors have tried different prior constraints on this factorization problem to enable them to get a unique solution to the low rank factorization. The most popular constraint (when the blind source separation problem is posed as a matrix factorization) is to assume that all three matrices are non-negative. This is known as non-negative matrix factorization (NNMF) [2] [3] [4] [5] [6]. Many problems are well suited to the NNMF approach. For example, an early work in this field took images of human face, where the pixels have positive values giving the intensity of the light for each color dimension. Using non-negative matrix factorization, it was shown that a dataset of faces could be decomposed into facial components [7], and that each face in the dataset had a sparse representation over these components, which seems to capture meaningful sub-features like the eyes, nose, mouth, etc. In [2] the authors assume that some of the signals are not blind, which makes the problem less ill posed. In [3] the authors impose a piece-wise constant prior, which assumes the power of the signal remains stable for a period of time. In [4] the authors study scenarios where different sources operate on different frequency bands. As we note below, even unmixing signal power may not unmix signal messages. Again, in [5], the authors are only interested in separating power spectra. In [6] the authors require there to be a prior on the signals, in that the statistics of the signals must be different.

For the blind source separation problem, when mixing signals from different communication transmitters, positive signals represent signal power [2] [3] [4] [5] [6]. This can be useful, especially when separating signals that exist over multiple frequency bands. However, there is a problem with using strictly power level, because it loses the amplitude information needed to separate the messages from each transmitter. This is because signals combine roughly linearly in amplitude, so that decomposing power information has a high error associated with that approach. The reason authors used the power approximation for communication signal separation, is that the model becomes more identifiable, since the non-negativity constraint reduces the degrees of freedom of the factorization. In this work, we will show we can also separate amplitudes in addition to power, so that the information contained in the signals may also be recovered.

Our approach will be based on taking the underdetermined factorization problem, and using the receiver locations, which would be available to the person measuring the incoming signals. Knowing the receiver locations reduces the degrees of freedom of the loss matrix U from quadratic (number of transmitters by number of receivers) to linear (number of transmitters). There are other groups that have used the receiver locations in different ways as well. One idea is to use the receiver locations to estimate the angle-of-arrival of the transmitted signals [1]. Our method is different, because angle-of-arrival is akin to a triangulation approach. Our method is instead based on a trilateration approach which we started developing in [8] and completed here with working models that obtain convergence.

There is also a body of knowledge about blind source separation that does not use the factorization approach.

Independent component analysis is based on statistical priors that mean the signals have some differences or that a statistical regularity makes the separation no longer undetermined [9]. There are many types of ICA that are based on different types of statistical priors. The advantage of our method is it does not assume any prior knowledge of the incoming signal characteristics.

# Method

We start with the existing matrix factorization model for blind source separation of communication signals, and add information from the receiver array configuration which would be available to the user measuring incoming signals. This additional information, if used as a constraint on an inverse problem, takes an underdetermined low rank factorization, and turns it into an overdetermined matrix factorization. This gives us a quality known as identifiability [10], which means that the solution is the unique combination of transmitted signals, and not any other combination that gives the received signals. approach is an example of solving an inverse problem. The inverse problem approach means that instead of taking an equation and trying to rearrange it to find a formula for a solution, you instead search over a space for a vector that makes the original equation true (possibly with a small error). This search is governed by considering a loss or error term that results from assuming different solution vectors. In general, the loss could incorporate more knowledge than just the error, because different errors may have different consequences, if we know how the information from the result will be applied [10]. This extended approach is known as decision theory and is not applied in our current model. However, this is how the model could be developed further in the future. For example, if you wanted to apply our method, and you knew a priori what modulation scheme a signal was using, you could penalize signals in the loss term that are divergence from known code words in the modulation scheme, or if you had a prior on the message code, you can even penalize over the entire message for combinations of symbols with low probability. This is one advantage of our method, is that it is compatible with decision theory as currently applied to signal processing in communication and estimation theory [11] [12].

Again, starting with eq. 1, we write the loss matrix U in terms of the distance between the source transmitters positions S and the receiver array positions R. S is an M×3 matrix for M different source signals and 3 coordinates of space x,y,z. R is an N×3 matrix for N different receivers and again 3 coordinates of space x,y,z. This is shown below in eq. 2 and hence U is an M×N matrix.

P=UQ=R1-S1-2R1-S2-2⋯R1-SN-2R2-S1-2R2-S2-2⋱R2-SN-2⋮  ⋮RM-S1-2RM-S2-2⋯RM-SN-2Q                                        (2)

And where each term of inverse distance squared is given explicitly as in eq. 3 below.

Rm-Sn-2=

Rm,x-Sn,x2+Rm,y-Sn,y2+Rm,z-Sn,z2-2=                                        (3)

Rm,x-Sn,x2+Rm,y-Sn,y2+Rm,z-Sn,z2-1

The R matrix is fully known, since the user making measurements knows the dimensions of the receiver array, and therefore U matrix is parameterized by the S matrix of unknown source transmitter locations. We find in practice that this parameterization allows us to recover the matrices U and Q from only the matrix of received mixed signals P.

We used two methods to solve the inverse problem given this formulation. First, we used the python scipy library least squares function. This involves defining a residuals vector function f, which we defined as in eq. 4 below. The least square solver takes the function f and an initial guess X=S,Q which we generate using a sample from a gaussian vector of independent components with 0 mean and unit variance per component. We note that we also normalized the values of constants R and P, which means the returned prediction of S and Q from the solver’s prediction of X=S,Q must be returned to the original coordinate system. To be completely explicit and avoid confusion, the function f gets the values of S and Q from X by indexing.

fX=P-UR,SXQX                                                                (4)

The second method that we used to solve the inverse problem is Adam optimizer in python Tensorflow. The method is based on constructing a loss function l and repeatedly performing a gradient descent in a loop for a given number of epochs (we used 50000) with a parameter estimation for each epoch again in the form X=S,Q. Eq. 5 below gives the form of the loss function l.

lX=P-UR,SXQXF2                                                       (5)

In practice we found that the least square residuals solver gives better performance, which we will provide next in the results section.

# Results and Discussion

First, we give a description of the experimental parameters on the source signals being transmitted and the receiver arrays being simulated.

Experiment design

Why pure gaussian noise

5 source transmitters

Array geometries

The receiver array is arranged in a cube. The number of slots on each side is varied from 4 to 6 (grid size), so that the array has 43=64, 53=125, or 63=216 receivers. With more receivers unmixing the sources becomes a more well determined problem. However, it also becomes computationally expensive in each pass of the solver. In many cases, there should be a balanced approach, where the algorithm can be implemented in real time, and the error is sufficiently low, given enough processing power (floating point operations per second, or appropriate parallelization).

Time signal window sizes (for real time approach, calculation number of samples, and convergence time vs information provided and larger matrix size making each iteration slow)

The code for the simulation and operating tolerance search can be found at [13].

Future work

Could extend this work by using an unsupervised clustering approach with variable number of clusters such as with a Chinese restaurant process model. There are also specific algorithms that can be combined with our approach that are designed to perform matrix factorization when the number of sources is unknow, for example the tuning-pruning model [14].

[1] Nikunen, Joonas, and Tuomas Virtanen. "Direction of arrival based spatial covariance model for blind sound source separation." IEEE/ACM transactions on audio, speech, and language processing 22.3 (2014): 727-739.

[2] Z. Hu et al., "Robust non-negative matrix factorization for joint spectrum sensing and

primary user localization in cognitive radio networks," in Waveform Diversity & Design

Conference (WDD), 2012 International, 2012, pp. 303-307: IEEE.

[3] A. Zaeemzadeh, M. Joneidi, B. Shahrasbi, and N. Rahnavard, "Missing spectrum-data

recovery in cognitive radio networks using piecewise constant nonnegative matrix

factorization," in Military Communications Conference, MILCOM 2015-2015 IEEE,

2015, pp. 238-243: IEEE.

[4] X. Fu, N. D. Sidiropoulos, and W.-K. Ma, "Power spectra separation via structured

matrix factorization," IEEE J. Sel. Topics Signal Process, 2015.

[5] X. Fu, N. D. Sidiropoulos, J. H. Tranter, and W.-K. Ma, "A factor analysis framework for

power spectra separation and multiple emitter localization," IEEE Transactions on Signal

Processing, vol. 63, no. 24, pp. 6581-6594, 2015.

[6] Kitamura, Daichi, et al. "Determined blind source separation unifying independent vector analysis and nonnegative matrix factorization." IEEE/ACM Transactions on Audio, Speech, and Language Processing 24.9 (2016): 1626-1641.

[7] Lee, Daniel D., and H. Sebastian Seung. "Learning the parts of objects by non-negative matrix factorization." nature 401.6755 (1999): 788-791.

[8] Glandon, Alexander M. "Recurrent neural networks and matrix methods for cognitive radio spectrum prediction and security." (2017).

[9] Cardoso, J-F. "Blind signal separation: statistical principles." Proceedings of the IEEE 86.10 (1998): 2009-2025.

[10] Fu, Xiao, et al. "Nonnegative matrix factorization for signal and data analytics: Identifiability, algorithms, and applications." IEEE Signal Process. Mag. 36.2 (2019): 59-80.

[11] Berger, James O. Statistical decision theory and Bayesian analysis. Springer Science & Business Media, 2013.

[12] Oppenheim, Alan V., and Ronald W. Schafer. Digital Signal Processing:(by) Alan V. Oppenheim (and) Ronald W. Schafer. Prentice-Hall, 1975.

[13] Glandon, Alexander M., Github Repo: https://github.com/abstract-enough/inverse-square-signal-separation, 2026

[14] Mørup, Morten, and Lars Kai Hansen. "Tuning pruning in sparse non-negative matrix factorization." 2009 17th European Signal Processing Conference. IEEE, 2009.

## Margins

If the paper does not have the margins shown in Table 1, it will not upload properly.

Table 1. Margins and print area specifications.

PAPER MARGINS

A4		Letter

Top margin	2.54 cm        (1in.)	Top margin	1.0 in.    (2.54 cm)

Bottom margin	4.94  cm (1.95 in.)	Bottom margin	1.25 in.    (3.17 cm)

Left, right margin	1.93 cm   (.76 in.)	Left, right margin	.88 in.    (2.22 cm)

## Fonts

Table 2 shows the font sizes and highlighting in a typical manuscript. These font "styles" are contained with this sample manuscript. Use Times Roman or another standard and embedded font to avoid font errors.

*aglan001@odu.edu; phone 757-404-7958

Table 2. Manuscript font sizes and formatting. The Microsoft Word template includes a pre-saved Style Set, which includes formatting styles below. This Style Set can be found under the Design menu as the first selected Style Set.

## Preformatted SPIE Template Styles

## Manuscript component 	Description

## SPIE paper title	16 pt. bold, center

## SPIE authors/affiliations	12 pt., center

Keywords: *Keywords*	10 pt., left justify

### SPIE ABSTRACT TITLE		11 pt. bOLD, center

SPIE abstract body text	10 pt., justify

### SPIE SECTION HEADING		11 pt. BOLD, center, whole numbers

1.1 Heading 2, SPIE subsection heading 	10 pt. bold, left justify, subsection numbers

SPIE body text	10 pt., justify

SPIE figure caption	9 pt., justify below figure

SPIE table caption	9 pt., justify above figure

SPIE footnote text	9 pt., justify, numbered

REFERENCE HEADING	11 pt. BOLD, center

1. SPIE reference listing	10 pt., justify, numbered consecutively

# Formatting of manuscript components

## Title

Center the paper title at the top of the page in 16-pt. bold. Only the first word, proper nouns, and acronyms are capitalized. Keep titles brief and descriptive. Spell out acronyms unless they are widely known. Avoid starting with articles or prepositions, e.g., “The study of … ,” or, “On the ….”

## Author affiliations

The author list is in 12-pt. regular, centered. Omit titles and degrees such as Dr., Prof., Ph.D., etc. The list of affiliations follows. Each author's affiliation should be clearly noted. Use superscript numerals or letters to denote connection between authors and affiliations.

## Section headings

Section headings are 11-pt. bold capitals, centered. Sections numbers have whole numbers, e.g., 1, 2, 3… Don't number the "Acknowledgements" and "References." Headings often used are: 1. Introduction, 2. Methodology, 3. Data, 4. Results, and 5. Conclusions.

## Subsection heading

Subsection headings are left-justified, 10-pt. bold. Capitalize the first word, acronyms, and proper nouns.

## Text

Text is 10 point and justified. Each sentence ends with a period and a single space before the next sentence.

## Figures and captions

Captions go below figures. Indent 5 spaces from left margin and justify.

Figure 1. Figure captions are indented 5 spaces and justified. If you are familiar with Word styles, you can insert a field code called Seq figure which automatically numbers your figures.

## Tables and captions

Tables are centered. The caption goes above the table. The caption text should indent 5 spaces from left margin and justify. Table examples are on page 1 and 2 and below.

# Multimedia figures – video and audio files

Video and audio files can be included for publication. Table 3 lists the specifications for the mulitimedia files. Use a screenshot or another illustration for placement in the text. Use the file name to begin the caption. The text of the caption must end with the text http://dx.doi.org/doi.number.goes.here which tells the SPIE editor where to insert the hyperlink in the digital version of the manuscript.

Here is a sample illustration and caption for a multimedia file:

Video 1. A label of “Video/Audio 1, 2, …” should appear at the beginning of the caption to indicate to which multimedia file it is linked . Include this text at the end of the caption: http://dx.doi.org/doi.number.goes.here

Table 3. Information on video and audio files that can accompany a manuscript submission.

| Item | Video | Audio |
| --- | --- | --- |
| File name | Video1, video2, … | Audio1, audio2, … |
| Number of files | 0-10 | 0-10 |
| Size of each file | Less than 100 mb | Less than 100 mb |
| File types accepted | .mpeg, .mp4, .mov (Quicktime), .wmv (Windows Media Player) | .wav, .mp4, .mp3 |

# Mathematical Equations

Use common fonts like Times Roman in your math equations. A math reference in a paragraph sentence such as  is not numbered. The steps of a mathematical argument can be numbered using a right-aligned tab for clarity, for example

α = 	(1)

and

ρ=.	(2)

# using THIS TEMPLATE AND ITS Automatic formatting

This document template and more information is available on SPIE.org: http://spie.org/manuscripts

or by emailing authorhelp@spie.org

## 5.1 Using SPIE styles

To use this template in Microsoft Word: open this file and save it to a new file name. Type over the existing text with your paper. Alternatively, you can delete all text and select the SPIE style for each manuscript element (i.e., title, heading, subheading, text body) by highlighting the text you wish to format, right click, select "Styles" and at the bottom of the Styles menu, select "Apply Styles." You will then be able to select your desired style from the drop-down menu.

Figure 2. Highlight the text you wish to format, right click, select "Styles" and at the bottom of the Styles menu, select "Apply Styles", and select desired style from pictured drop-down menu.

Acknowledgements

Use this section to identify people who have aided the authors in accomplishing the work presented and to acknowledge sources of funding. Include grant numbers and the full name of the funding body.

References

Author(s), “Paper, Poster, or Presentation Title,” Volume Number, Volume Title, CID/Paper Number (Publication Date); DOI URL

Sarah Everton, Phill Dickens, Chris Tuck, Ben Dutton, “Evaluation of laser ultrasonic testing for inspection of metal additive manufacturing," Proc. SPIE 10352, Laser 3D Manufacturing II, 1035216 (16 March 2022); http://dx.doi.org/10.1117/12.2078768

Author(s), “Paper,” Journal Name. Volume Number(Issue Number), CID (Publication Date). DOI URL

Blackberrie Eddins and Muyinatu A. Lediju Bell, “Design of a multifiber light delivery system for photoacoustic-guided surgery," J. Biomed. Opt. 22(4), 041011 (13 Jan 2022). http://dx.doi.org/10.1117/1.JBO.22.4.041011

Booth, N. and Smith, A. S., [Infrared Detectors], Goodwin House Publishers, New York & Boston, 241-248 (1997).

Davis, A. R., Bush, C., Harvey, J. C. and Foley, M. F., "Fresnel lenses in rear projection displays," SID Int. Symp. Digest Tech. Papers 32(1), 934-937 (2001). http://dx.doi.org/10.0011/10.000000